Departure Checklist

Companionway hatch boards removed & stowed

Cockpit lockers unlocked

Check packing gland (1-2 drops/min)

Check engine oil level

Check gear oil level

Check coolant level

Cooling water thru-hull OPEN, hose & clamps OK

Check for fuel spill in bilge

Lav (sink/toilet) thru-hulls OPEN, hoses & clamps OK

Galley thru-hull OPEN, hose & clamps OK

Battery to #1 (odd date) or #2 (even date)

Check bilge pump operation (switch to “manual”)

AC breaker OFF; Battery Charger OFF

DC switches ON: Cabin lights, Refrigeration, Strereo,

VHF, Nav Inst, Autopilot, Bilge Pump

VHF Radio ON (check Wx, then Ch 16 & Wx)

Binnacle cover removed & stowed

Key ON, Blower ON, Engine Shutoff DOWN

Transmission NEUTRAL; Throttle @ 1/4

Start engine (max 30 seconds cranking)

Check exhaust water return, check gauges, rpm = 900

Dockside AC OFF; Remove shorepower cord

Check Standing Rigging (damage, wear, tension)

Halyards & Sheets untangled

Cockpit cushions in place

Dodger window covers removed & stowed

Check wheel for free movement; Center wheel

Have PFD's Available / Brief Passengers on Safety

Plan Departure

Cast off docklines

Stow fenders

Float Plan Filed?

Arrival Checklist

Fenders positioned; Boathook ready

Docklines secured

Idle at 900rpm for 3-5 minutes

Pull engine shutoff UP then key OFF

Reconnect shorepower cable

Cockpit cushions stowed

Binnacle cover in place

Dodger window covers in place

Secure halyards off mast

Battery to BOTH (engine must be OFF)

AC breaker ON, Charger ON

All DC switches OFF except: Bilge Pump, Refrig

Thru-hulls CLOSED: Lav sink, Galley sink, Macerator

Check for fuel spill in bilge

CLOSE cooling water thru-hull

Check packing gland leak rate

Wipe Down

Lock All Lockers & Companionway

Check Cockpit Drains Clear

Rough Weather

PFD's On

Jackline & Harness as required

All Hatches & Portholes Secured

Check Cockpit Drains Clear

Secure Loose Items Below

Check Bilge Pump operational

Companionway Closed & Secured

Anchor ready